| Lab: Lab 4 – VHDL Design of ALU and its FPGA Implementation |  |
| --- | --- |
| Name: Solomon Ewusi | Student ID: N01659373 |
| Date: October 19, 2026 |  |

Please refer to the lab manual posted on the blackboard and fill in the blanks in the following table as your lab report.

Notice: All labs are individual work and submission. All academic misconduct behaviors will be penalized to the maximum extent possible according to college policy. Thanks for your attention. (Reference Link: About Academic Misconduct)

Fill in all the lab report submission requirements in the following table. Please add more rows if necessary.

| Requirements | Your submissions (Answers/Source Code/Screenshots/File Submissions, etc.) Please notice: If it is required to submit a separate file, please attach it separately on the blackboard. |
| --- | --- |
| Professor’s demo signature marked on the attendance sheet? (demo the mux code in the first week and demo the circuit testing on the training board) | The ALU circuit was implemented and tested on the BASYS 3 FPGA training board. Photos of the board test results for all 8 functions are included in the last row of this table as proof of the demonstration. |
| Your VHDL source codes (ALU.vhd and Mux.vhd) | ALU.vhd and Mux.vhd are submitted as separate files on Blackboard. |
| Screen shots of simulation runs for at least 360ms (please make the screen in full window size by clicking the button). | Screenshot 1: Full behavioral simulation, 0 – 360 ms (values in hexadecimal) Screenshot 2: Same simulation with A, B and RSLT shown in binary Explanation: The testbench (WrapperTb.vhd) was run for 360 ms, as shown by "XSim simulation ran for 360ms" in the Tcl Console. For the first 160 ms, A = 101001 and B = 001110 are held constant while S steps through all 8 codes (20 ms each), so every ALU function is tested once: S; Function; RSLT (binary); Decimal. 000; A + B; 110111; −9. 001; A − B; 011011; 27. 010; A AND B; 001000; 8. 011; A OR B; 101111; −17. 100; A XOR B; 100111; −25. 101; NOT A; 010110; 22. 110; Shift A right; 010100; 20. 111; Shift A left; 010010; 18. From 160 ms to 360 ms the testbench repeats add (000) and subtract (001) with new input pairs to check the condition flags. For example, 011111 − 011111 = 000000 sets ZF = 1, and 100000 + 000000 = 100000 sets NF = 1. All simulated results match the expected values, which confirms the ALU works correctly. |
| Final results of the circuit testing on the FPGA board. S (SW 2 to 0); A (SW 15 to 10); B (SW 9 to 4); Display on LEDs. 000; 110100; 111010; −18 (LEDs 101110). 001; 110100; 111010; −6 (LEDs 111010). 010; 110100; 111010; −16 (LEDs 110000). 011; 110100; 111010; −2 (LEDs 111110). 100; 110100; 111010; 14 (LEDs 001110). 101; 110100; 111010; 11 (LEDs 001011). 110; 110100; 111010; 26 (LEDs 011010). 111; 110100; 111010; −24 (LEDs 101000). Board test photos Inputs set on the switches for every test: A = 110100 (SW15–SW10), B = 111010 (SW9–SW4); only S (SW2–SW0) changes. LD15–LD10 show RSLT in binary, LD3–LD0 show CF, NF, OVF and ZF, and the 7-segment display shows RSLT as a signed decimal number. All 8 results on the board match the simulation and the expected values. 000 – A + B: display −18; 001 – A − B: display −6. 010 – A AND B: display −16; 011 – A OR B: display −2. 100 – A XOR B: display 14; 101 – NOT A: display 11. 110 – Shift A right: display 26; 111 – Shift A left: display −24. |